More Stuff About the FDE

by Daniel Singer - L1011-1 (Delta Airlines - N729DA)

1.	Any problems with responsiveness have been fixed

2.	Flap notches were corrected from 9 to 8 (including up)

3.	Initial climb out at anywhere between 1,500 - 2,000 feet per minute

4.	Take off with flaps at 3 notches (18 degrees) and trim at about +5 @ 94% N1

5.	Retract flaps to 10 degrees at about 180 knots IAS

6.	Retract flaps to 4 degrees at about 195 knots IAS

7.	Retract flaps to 0 degrees (UP) at about 205 knots

8. 	Maintain 250 knots to FL100

9.	At FL100, reduce climb rate to 1,500 feet per minute

10.	Speed up to 300 knots IAS

11.	At FL140 speed up to 320 knots IAS

12.	Set initial crusing altitude at either FL290 or FL310

13.	Do not go higher than FL350

14. 	Cruise at M.79 for long range and M.80 for short range

15.	On landing, pitch up will be about 5 degrees; this is realistic!

16.	Maximum Landing Weight - 348,000 pounds

17. 	This means you can not land with more than 77,500 lbs. of fuel

18. 	In this model, tanks CAN be all filled to 100% - this is MTOW - 430,000 pounds - there was a huge 	outcry for a true empy weight...so here it is.

19.	The RB engines are 3 stage...this means they have an N3 gauge and spool up very slowly.

20.	Use all enclosed charts for exact speeds.

Special Thanks Go To:

Ryan Suetta

Kamal Kerba

Darim Rahmatallah

Noel Benford

Christine Derksen/PF

Chris Coduto

Chris Kilroy

Bryan Shrode